SECTION 33 49 23

STORM DRAINAGE WATER RETENTION STRUCTURES

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Storm drainage water retention structures.

1.2	RELATED SECTIONS

A.	Section 31 23 00 - Excavation and Fill.

B.	Section 33 41 00 – Storm Utility Drainage Piping.

1.3	SUBMITTALS

A.	Comply with Section 01 33 00 - Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, including installation instructions.

C.	Shop Drawings:  Submit manufacturer's shop drawings, indicating materials, components, and dimensions.

D.	Warranty:  Submit manufacturer's standard warranty.

1.4	QUALITY ASSURANCE

A.	Single Source Responsibility:  Provide storm water leaching system from single manufacturer.

1.5	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area in accordance with manufacturer's instructions.

C.	Handling:  Protect materials during handling and installation to prevent damage.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	NDS, Inc., 851 North Harvard Avenue, PO Box 339, Lindsay, California 93247.  Toll Free (800) 726-1994.  Toll Free Fax (800) 726-1998.  Phone (559) 562-9888.  Fax (559) 562-4488.  Website www.ndspro.com.  E-Mail nds@ndspro.com.

2.2	FLO-WELL STORM DRAINAGE WATER RETENTION STRUCTURE

A.	24-Inch Diameter by 28.75-Inch High Flo-Well Storm Water Leaching System:

1.	Part Number:  FWAS24.

2.	Description:  Structural-foam polyolefin, round drywell system with UV inhibitor.

3.	Diameter:  24 inches.

4.	Height:  28.75 inches.

5.	Capacity:  49 gallons.

6.	Knockout Leaching Ports per Panel:  18, 1.5-inch diameter.

7.	Knockout Inlet/Outlet Ports per Panel:  3, 4.5-inch diameter.

8.	Color:  Black.

B.	24-Inch Diameter Flo-Well Cover:

1.	Part Number:  FWAS24C.

2.	Description:  Structural-foam polyolefin, round drywell system cover with UV inhibitor.

3.	Diameter:  24-1/4 inches.

4.	Height:  4-5/8 inches.

5.	Outer Lip:  3/4 inch.

6.	Center Knockout:  4.5-inch diameter.

7.	Color:  Black.

C.	Flo-Well Side Panels/Extension Only:

1.	Part Number:  FWSPS3.

2.	Description:  Structural-foam polyolefin, side panels with UV inhibitor.

3.	Size:  24 inches high with recessed 0.75-inch lip.

4.	Knockout Leaching Ports per Panel:  18, 1.5-inch diameter.

5.	Knockout Inlet/Outlet Ports per Panel:  3, 4.5-inch diameter.

6.	Color:  Black.

D.	24-Inch Diameter Flo-Well Bottom:

1.	Part Number:  FWBP24.

2.	Description:  Structural-foam polyolefin, round drywell system bottom with UV inhibitor.

3.	Diameter:  24 inches.

4.	Leaching Ports:  6, 1-1/2-inch diameter.

5.	Center Knockout for Strut Coupling:  4.5-inch diameter.

6.	Color:  Black.

E.	4-Inch Schedule 40 Surface Drain Inlet with Grate:

1.	Part Number:  FWSD69.

2.	Description:  Structural-foam polyolefin, 6-inch grate with 4-inch inlet with UV inhibitor.

3.	Diameter at Inlet:  6 inches.

4.	Height:  9 inches.

5.	Outside Diameter:  4.5 inches, fits Schedule 40 PVC fittings.

6.	Color:  Black.

F.	Porous Filter Fabric Wrap for Flo-Well:

1.	Part Number:  FWFF67.

2.	Description:  7-foot long by 2-foot wide, non-woven fabric to surround Flo-Well drywell system.

3.	Weight:  1 ounce.

4.	Mullen Burst:  175 psi.

5.	Flow Through:  200 gpm.

6.	Color:  Black.

2.3	OPTIONS

A.	9-Inch by 9-Inch Atrium Grate:

1.	Part Number: 981.

2.	Description:  9-inch square, structural-foam polyolefin, dome atrium grate with UV inhibitor.

3.	Grate Openings:  3/8 inch.

4.	Open Surface Area:  30.28 square inches.

5.	Inlet Capacity:  39.67 gpm.

6.	Color: Black.

B.	9-Inch Square Grate:

1.	Part Number:  980.

2.	Description:  9-inch square, structural-foam polyolefin, grate with UV inhibitor.

3.	Grate Openings:  7/16 inch.

4.	Open Surface Area:  39.50 square inches.

5.	Inlet Capacity:  51.75 gpm.

6.	Load:  61 to 175 psi.

7.	Color:  Black.

C.	12-Inch by 12-Inch Atrium Grate:

1.	Part Number:  1280.

2.	Description:  12-inch square, structural-foam polyolefin, dome atrium grate with UV inhibitor.

3.	Grate Openings:  3/8 inch.

4.	Open Surface Area:  60.00 square inches.

5.	Inlet Capacity:  78.60 gpm.

6.	Color:  Black.

D.	12-Inch by 12-Inch Square Grate:

1.	Part Number:  1211.

2.	Description:  12-inch square, structural-foam polyolefin, grate with UV inhibitor.

3.	Grate Openings:  3/8 inch.

4.	Open Surface Area:  50.76 square inches.

5.	Inlet Capacity:  66.50 gpm.

6.	Load:  61 to 175 psi.

7.	Color:  Black.

E.	9-Inch by 9-Inch Low-Profile Adapter:

1.	Part Number:  930.

2.	Description:  9-inch by 9-inch, low-profile adapter with bayonet bottom outlet.

3.	Color:  Black.

F.	12-Inch by 12-Inch Low-Profile Adapter:

1.	Part Number:  1230.

2.	Description:  12-inch by 12-inch, low-profile adapter with bayonet bottom outlet.

3.	Color:  Black.

2.3	OPTIONS (CONT.)

G.	3-Inch and 4-Inch Universal Locking Outlet:

1.	Part Number:  1243.

2.	Description:  3-inch and 4-inch universal locking outlet.

3.	Color:  Black.

H.	4-inch Schedule 40 pipe.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive storm water leaching system.  Notify Architect if areas are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.2	INSTALLATION

A.	Install storm water leaching system in accordance with manufacturer's instructions at locations indicated on the Drawings.

B.	Install leaching system level and to proper elevation.

3.3	PROTECTION

A.	Protect installed storm water leaching system from damage during construction.

END OF SECTION

Last Edited: 03/01/2010