SECTION 32 94 13

LANDSCAPE EDGING – ROOT BARRIER SHEET MATERIAL

PART 1	GENERAL

1.1	SECTION INCLUDES

A.	Landscape Edging - Root Barrier Sheet Material.

1.2	RELATED SECTIONS

A.	Section 32 93 00 - Plants.

1.3	REFERENCES

A.	ASTM D 256 - Standard Test Methods for Determining the Izod Pendulum Impact Resistance of Plastics.

B.	ASTM D 638 - Standard Test Method for Tensile Properties of Plastics.

C.	ASTM D 785 - Standard Test Method for Rockwell Hardness of Plastics and Electrical Insulating Materials.

D.	ASTM D 790 - Standard Test Methods for Flexural Properties of Unreinforced and Reinforced Plastics and Electrical Insulating Materials.

1.4	SUBMITTALS

A.	Comply with Section 01 33 00 - Submittal Procedures.

B.	Product Data:  Submit manufacturer's product data, including installation instructions.

C.	Samples:  Submit manufacturer's 6-inch by 6-inch sample of root barrier sheet material.

D.	Warranty:  Submit manufacturer's standard warranty.

1.5	DELIVERY, STORAGE, AND HANDLING

A.	Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

B.	Storage:  Store materials in clean, dry area protected from weather in accordance with manufacturer's instructions.  Do not store materials in direct sunlight.

C.	Handling:  Protect materials during handling and installation to prevent damage.

PART 2	PRODUCTS

2.1	MANUFACTURER

A.	NDS, Inc., 851 North Harvard Avenue, PO Box 339, Lindsay, California 93247.  Toll Free (800) 726-1994.  Toll Free Fax (800) 726-1998.  Phone (559) 562-9888.  Fax (559) 562-4488.  Website www.ndspro.com.  E-Mail nds@ndspro.com.

2.2	ROOT BARRIER SHEET MATERIAL

A.	Root Barrier Sheet Material:  Shawtown SM Series root barrier sheet material.

1.	12-Inch High by 20-Foot Long SM Rolls:  Part Number SM-1220.

2.	18-Inch High by 20-Foot Long SM Rolls:  Part Number SM-1820.

3.	24-Inch High by 20-Foot Long SM Rolls:  Part Number SM-2420.

4.	36-Inch High by 20-Foot Long SM Rolls:  Part Number SM-3620.

5.	48-Inch High by 20-Foot Long SM Rolls:  Part Number SM-4820.

B.	Description:

1.	1-piece, without hinge-type sections.

2.	Material:  Up to 100 percent post-consumer, high-impact polystyrene (HIPS) with rubberizer added and UV inhibitor.

3.	Thickness:  Minimum 0.040” - 0.060”.

4.	Connections:  Stapled Overlap.

5.	Reinforcing Ribs:  1/2-inch deep, raised vertical ribs running perpendicular to sheet, 6 inches on center.

6.	Color:  Black.

C.	Physical Properties:

1.	Tensile Stress, ASTM D 638:  3,800 psi.

2.	Yield Elongation, ASTM D 638:  35 percent.

3.	Flexural Modulus, ASTM D 790:  300,000 psi.

4.	Notched Izod Impact, ASTM D 256:  2.0 psi at 73 degrees F.

5.	Rockwell Hardness, ASTM D 785:  L50.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Examine areas to receive root barrier sheet material.  Notify Architect if areas are not acceptable.  Do not begin installation until unacceptable conditions have been corrected.

3.2	INSTALLATION

A.	Install root barrier sheet material in accordance with manufacturer's instructions at locations indicated on the Drawings.

3.2	INSTALLATION (CONT.)

B.	Linear/Root Pruning Application:  Place root barrier sheet material directly adjacent to hardscape being protected.

C.	Surround Application:  Place root barrier sheet material around root ball, at sufficient distance to allow proper backfill.

D.	Do not allow roots to circle or grow over top of root barrier sheet material.

END OF SECTION

Last Edited: 03/01/2010